#include "ti_msp_dl_config.h"

#include "JY901S.h"

#include "Sensor.h"

#include "PWM.h"

#include "LED.h"

#include "Buzzer.h"

#include "Encoder.h"

#include <stdint.h>

/* ===================== 基础速度参数 ===================== */

#define BASE_SPEED                  300     // A->B / C->D 无黑线直线速度

/*

* 这里接近第一个代码的循迹速度。

* 如果太快，可以改回 220。

*/

#define ARC_SPEED                   250     // B->C / D->A 右半圆循迹速度

#define MAX_TIME_MS                 30000U

/* ===================== 编码器参数 ===================== */

#define COUNTS_PER_CM               20

#define STRAIGHT_TO_ARC_MIN_CM      70

#define AB_BACKUP_DISTANCE_CM       108

#define CD_BACKUP_DISTANCE_CM       108

#define ARC_MIN_DISTANCE_CM         90

#define ARC_FALLBACK_DISTANCE_CM    125

/*

* 这个保留，但现在 B->C / D->A 结束主要用丢线判断。

*/

#define ARC_YAW_TARGET_DEG          172.0f

/*

* C 点出弯后往内侧多修正约 10 度。

* 如果修反了，就改成 -10.0f。

*/

#define C_OUT_BIAS_DEG              3.0f

/* ===================== 直线修正参数 ===================== */

#define ENCODER_KP                  1

#define YAW_KP                      2

#define MAX_ENCODER_CORRECTION      60

#define MAX_YAW_CORRECTION          50

#define MAX_STRAIGHT_CORRECTION     100

/* ===================== 灰度循迹参数 ===================== */

/*

* 这里用第一个代码的循迹方式：

* correction = line_error * LINE_KP_ARC

*/

#define LINE_KP_ARC                 60

#define MAX_LINE_CORRECTION_ARC     180

#define LINE_ON_CONFIRM             2

#define LINE_LOST_CONFIRM           5U

/* ===================== C 点轻微摆正参数 ===================== */

#define ALIGN_FORWARD_SPEED         130

#define ALIGN_KP                    4

#define ALIGN_MAX_CORRECTION        32

#define ALIGN_OK_DEG                5.0f

#define ALIGN_MAX_TIME_MS           320

/*

* 如果 C 点摆正时越修越歪，把 0 改成 1。

*/

#define ALIGN_REVERSE               1

/* ===================== 全局变量 ===================== */

static volatile uint32_t g_ms = 0;

typedef enum

{

STATE_A_TO_B = 0,

STATE_B_TO_C,

STATE_C_ALIGN_TO_D,

STATE_C_TO_D,

STATE_D_TO_A,

STATE_STOP

} CarState_t;

static CarState_t g_state = STATE_A_TO_B;

static uint32_t g_start_time = 0;

static uint32_t g_state_time = 0;

static float g_yaw_start = 0.0f;

static int8_t g_last_line_error = 0;

static uint8_t g_line_on_count = 0;

static uint8_t g_line_lost_count = 0;

/*

* C 点出弯摆正目标角度。

* 单独保存，避免 yaw 过 180° 后正负号跳变导致修正方向乱掉。

*/

static float g_c_align_target = 180.0f;

/* ===================== Debug 监视变量 ===================== */

volatile uint8_t debug_state = 0;

volatile uint8_t debug_gray = 0;

volatile uint8_t debug_line_hit_count = 0;

volatile int8_t debug_line_error = 0;

volatile int16_t debug_left_speed = 0;

volatile int16_t debug_right_speed = 0;

volatile int32_t debug_left_encoder = 0;

volatile int32_t debug_right_encoder = 0;

volatile int32_t debug_avg_encoder = 0;

volatile float debug_yaw = 0.0f;

volatile float debug_relative_yaw = 0.0f;

volatile float debug_align_error = 0.0f;

/* ===================== SysTick ===================== */

void SysTick_Handler(void)

{

g_ms++;

}

static void Delay_ms(uint32_t ms)

{

uint32_t start = g_ms;

while ((g_ms - start) < ms)

{

JY901S_Update();

}

}

/* ===================== 工具函数 ===================== */

static int32_t Abs32(int32_t x)

{

return (x < 0) ? -x : x;

}

static float AbsFloat(float x)

{

return (x < 0.0f) ? -x : x;

}

static int16_t Limit_Int16(int32_t value, int16_t min, int16_t max)

{

if (value > max)

{

return max;

}

if (value < min)

{

return min;

}

return (int16_t)value;

}

static float Normalize_Angle(float angle)

{

while (angle > 180.0f)

{

angle -= 360.0f;

}

while (angle < -180.0f)

{

angle += 360.0f;

}

return angle;

}

static int32_t Get_Avg_Encoder(void)

{

int32_t left;

int32_t right;

left = Abs32(Encoder_GetLeft());

right = Abs32(Encoder_GetRight());

return (left + right) / 2;

}

static uint8_t Distance_OK(uint16_t cm)

{

int32_t target_count;

target_count = (int32_t)cm * COUNTS_PER_CM;

return (Get_Avg_Encoder() >= target_count);

}

static float Get_Relative_Yaw(void)

{

JY901S_Update();

if (!JY901S_HasYaw())

{

return 0.0f;

}

return Normalize_Angle(JY901S_GetYaw() - g_yaw_start);

}

/* ===================== 声光提示 ===================== */

static void Alert_Once(void)

{

Motor_Stop();

LED_On();

Buzzer_On();

Delay_ms(120);

LED_Off();

Buzzer_Off();

Delay_ms(80);

}

/* ===================== 状态切换 ===================== */

static void Begin_State(CarState_t next)

{

g_state = next;

g_state_time = g_ms;

Encoder_Clear();

g_last_line_error = 0;

g_line_on_count = 0;

g_line_lost_count = 0;

JY901S_Update();

if (JY901S_HasYaw())

{

g_yaw_start = JY901S_GetYaw();

}

debug_state = (uint8_t)g_state;

}

/* ===================== 七路灰度 ===================== */

/*

* Gray_Read_All() 必须返回：

*

* bit6 bit5 bit4 bit3 bit2 bit1 bit0

* L3   L2   L1   M    R1   R2   R3

*

* 黑线 = 1

* 白底 = 0

*/

static uint8_t Count_Gray_Hit(uint8_t gray)

{

uint8_t count = 0;

for (uint8_t i = 0; i < 7; i++)

{

if (gray & (1 << i))

{

count++;

}

}

return count;

}

static uint8_t Is_Line_Detected(void)

{

uint8_t gray;

gray = Gray_Read_All();

debug_gray = gray;

debug_line_hit_count = Count_Gray_Hit(gray);

return (debug_line_hit_count >= 1);

}

static uint8_t Line_On_Stable(void)

{

if (Is_Line_Detected())

{

if (g_line_on_count < LINE_ON_CONFIRM)

{

g_line_on_count++;

}

}

else

{

g_line_on_count = 0;

}

return (g_line_on_count >= LINE_ON_CONFIRM);

}

static uint8_t Line_Lost_Stable(void)

{

if (!Is_Line_Detected())

{

if (g_line_lost_count < LINE_LOST_CONFIRM)

{

g_line_lost_count++;

}

}

else

{

g_line_lost_count = 0;

}

return (g_line_lost_count >= LINE_LOST_CONFIRM);

}

/*

* A->B / C->D 直线末端判断：

* 走到 B/D 附近后，灰度看到半圆黑线就切换。

*/

static uint8_t Straight_To_Arc_End_OK(uint16_t backup_cm)

{

if (Distance_OK(STRAIGHT_TO_ARC_MIN_CM) && Line_On_Stable())

{

return 1;

}

if (Distance_OK(backup_cm))

{

return 1;

}

return 0;

}

/* ===================== 无黑线直线控制 ===================== */

static void Straight_Run_No_Line(uint16_t base_speed)

{

int32_t left_count;

int32_t right_count;

int32_t encoder_error;

int32_t encoder_correction;

int32_t yaw_correction;

int32_t correction;

int16_t left_speed;

int16_t right_speed;

float yaw_error;

JY901S_Update();

left_count = Abs32(Encoder_GetLeft());

right_count = Abs32(Encoder_GetRight());

encoder_error = left_count - right_count;

encoder_correction = encoder_error * ENCODER_KP;

encoder_correction = Limit_Int16(

encoder_correction,

-MAX_ENCODER_CORRECTION,

MAX_ENCODER_CORRECTION

);

yaw_correction = 0;

if (JY901S_HasYaw())

{

yaw_error = Get_Relative_Yaw();

yaw_correction = (int32_t)(yaw_error * YAW_KP);

yaw_correction = Limit_Int16(

yaw_correction,

-MAX_YAW_CORRECTION,

MAX_YAW_CORRECTION

);

}

correction = encoder_correction + yaw_correction;

correction = Limit_Int16(

correction,

-MAX_STRAIGHT_CORRECTION,

MAX_STRAIGHT_CORRECTION

);

/*

* 如果无黑线直线修正方向反了，

* 就把下面两行的 + / - 对调。

*/

left_speed = base_speed - correction;

right_speed = base_speed + correction;

left_speed = Limit_Int16(left_speed, 0, 1000);

right_speed = Limit_Int16(right_speed, 0, 1000);

Motor_SetSpeed(left_speed, right_speed);

debug_left_speed = left_speed;

debug_right_speed = right_speed;

}

/* ===================== 灰度循迹 ===================== */

/*

* L3 L2 L1 M R1 R2 R3

* -3 -2 -1 0 +1 +2 +3

*

* line_error > 0：黑线在右侧，左轮快，右轮慢，车向右修。

* line_error < 0：黑线在左侧，左轮慢，右轮快，车向左修。

*/

static int8_t Get_Line_Error(uint8_t gray)

{

int16_t sum = 0;

int8_t count = 0;

if (gray & (1 << 6)) { sum += -3; count++; }  // L3

if (gray & (1 << 5)) { sum += -2; count++; }  // L2

if (gray & (1 << 4)) { sum += -1; count++; }  // L1

if (gray & (1 << 3)) { sum +=  0; count++; }  // M

if (gray & (1 << 2)) { sum +=  1; count++; }  // R1

if (gray & (1 << 1)) { sum +=  2; count++; }  // R2

if (gray & (1 << 0)) { sum +=  3; count++; }  // R3

/*

* 没检测到黑线就返回 0。

* 这样不会突然猛打方向。

*/

if (count == 0)

{

return 0;

}

return (int8_t)(sum / count);

}

static void Line_Follow_RightArc(uint16_t base_speed)

{

uint8_t gray;

int8_t line_error;

int32_t correction;

int16_t left_speed;

int16_t right_speed;

JY901S_Update();

gray = Gray_Read_All();

debug_gray = gray;

debug_line_hit_count = Count_Gray_Hit(gray);

line_error = Get_Line_Error(gray);

g_last_line_error = line_error;

correction = (int32_t)line_error * LINE_KP_ARC;

correction = Limit_Int16(

correction,

-MAX_LINE_CORRECTION_ARC,

MAX_LINE_CORRECTION_ARC

);

left_speed = base_speed + correction;

right_speed = base_speed - correction;

left_speed = Limit_Int16(left_speed, 0, 1000);

right_speed = Limit_Int16(right_speed, 0, 1000);

Motor_SetSpeed(left_speed, right_speed);

debug_line_error = line_error;

debug_left_speed = left_speed;

debug_right_speed = right_speed;

}

/* ===================== 半圆结束判断 ===================== */

static uint8_t Arc_Min_Distance_OK(void)

{

return Distance_OK(ARC_MIN_DISTANCE_CM);

}

/*

* 现在真正用于 B->C / D->A 的结束判断：

* 先走够弧线最小距离，再连续几次检测不到黑线。

*/

static uint8_t Arc_End_By_LineLost_OK(void)

{

if (Arc_Min_Distance_OK() && Line_Lost_Stable())

{

return 1;

}

return 0;

}

/* ===================== C 点出弯目标角度设置 ===================== */

static void Set_C_Align_Target(void)

{

float yaw;

yaw = Get_Relative_Yaw();

/*

* 如果 B->C 半圆转向后 yaw 是正方向，

* 目标就是 180° 再往内侧多修。

*

* 如果 yaw 是负方向，

* 目标就是 -180° 再往内侧多修。

*/

if (yaw >= 0.0f)

{

g_c_align_target = Normalize_Angle(180.0f + C_OUT_BIAS_DEG);

}

else

{

g_c_align_target = Normalize_Angle(-180.0f - C_OUT_BIAS_DEG);

}

}

/* ===================== C 点轻微前进摆正 ===================== */

static uint8_t Align_To_Arc_180(void)

{

float yaw;

float target;

float error;

int32_t correction;

int16_t left_speed;

int16_t right_speed;

JY901S_Update();

yaw = Get_Relative_Yaw();

/*

* 使用进入 C_ALIGN 状态时保存好的目标角度。

*/

target = g_c_align_target;

error = Normalize_Angle(target - yaw);

debug_align_error = error;

if (AbsFloat(error) <= ALIGN_OK_DEG)

{

Motor_Stop();

return 1;

}

if ((g_ms - g_state_time) >= ALIGN_MAX_TIME_MS)

{

Motor_Stop();

return 1;

}

correction = (int32_t)(error * ALIGN_KP);

correction = Limit_Int16(

correction,

-ALIGN_MAX_CORRECTION,

ALIGN_MAX_CORRECTION

);

#if ALIGN_REVERSE

left_speed  = ALIGN_FORWARD_SPEED - correction;

right_speed = ALIGN_FORWARD_SPEED + correction;

#else

left_speed  = ALIGN_FORWARD_SPEED + correction;

right_speed = ALIGN_FORWARD_SPEED - correction;

#endif

left_speed = Limit_Int16(left_speed, 0, 1000);

right_speed = Limit_Int16(right_speed, 0, 1000);

Motor_SetSpeed(left_speed, right_speed);

debug_left_speed = left_speed;

debug_right_speed = right_speed;

return 0;

}

/* ===================== Debug 更新 ===================== */

static void Debug_Update(void)

{

JY901S_Update();

debug_state = (uint8_t)g_state;

debug_left_encoder = Encoder_GetLeft();

debug_right_encoder = Encoder_GetRight();

debug_avg_encoder = Get_Avg_Encoder();

if (JY901S_HasYaw())

{

debug_yaw = JY901S_GetYaw();

debug_relative_yaw = Get_Relative_Yaw();

}

}

/* ===================== 主函数 ===================== */

int main(void)

{

SYSCFG_DL_init();

SysTick_Config(CPUCLK_FREQ / 1000);

Gray_Init();

Motor_Init();

LED_Init();

Buzzer_Init();

Encoder_Init();

JY901S_Init();

Encoder_Clear();

/*

* 等 JY901S 稳定。

*/

for (uint16_t i = 0; i < 100; i++)

{

JY901S_Update();

Delay_ms(5);

}

if (JY901S_HasYaw())

{

g_yaw_start = JY901S_GetYaw();

}

else

{

g_yaw_start = 0.0f;

}

g_start_time = g_ms;

g_state_time = g_ms;

g_state = STATE_A_TO_B;

debug_state = (uint8_t)g_state;

while (1)

{

JY901S_Update();

Debug_Update();

if ((g_state != STATE_STOP) &&

((g_ms - g_start_time) >= MAX_TIME_MS))

{

Motor_Stop();

Alert_Once();

g_state = STATE_STOP;

}

switch (g_state)

{

case STATE_A_TO_B:

/*

* A -> B：没有黑线。

* 编码器 + 陀螺仪走直线。

*/

Straight_Run_No_Line(BASE_SPEED);

if (Straight_To_Arc_End_OK(AB_BACKUP_DISTANCE_CM))

{

Alert_Once();

Begin_State(STATE_B_TO_C);

}

break;

case STATE_B_TO_C:

/*

* B -> C：灰度循迹。

* 结束条件：走够最小弧线距离 + 检测不到黑线。

*/

Line_Follow_RightArc(ARC_SPEED);

if (Arc_End_By_LineLost_OK())

{

/*

* 到 C 点检测不到黑线，立刻声光提示。

*/

Alert_Once();

/*

* 进入 C 点摆正状态前，保存目标角度。

*/

Set_C_Align_Target();

g_state = STATE_C_ALIGN_TO_D;

g_state_time = g_ms;

debug_state = (uint8_t)g_state;

}

break;

case STATE_C_ALIGN_TO_D:

/*

* 出 C 点后低速向前摆正。

*/

if (Align_To_Arc_180())

{

Begin_State(STATE_C_TO_D);

}

break;

case STATE_C_TO_D:

/*

* C -> D：没有黑线。

*/

Straight_Run_No_Line(BASE_SPEED);

if (Straight_To_Arc_End_OK(CD_BACKUP_DISTANCE_CM))

{

Alert_Once();

Begin_State(STATE_D_TO_A);

}

break;

case STATE_D_TO_A:

/*

* D -> A：灰度循迹。

* 结束条件：走够最小弧线距离 + 检测不到黑线。

*/

Line_Follow_RightArc(ARC_SPEED);

if (Arc_End_By_LineLost_OK())

{

Alert_Once();

g_state = STATE_STOP;

debug_state = (uint8_t)g_state;

}

break;

case STATE_STOP:

default:

Motor_Stop();

break;

}

Delay_ms(10);

}

}