#include "ti_msp_dl_config.h"

#include "JY901S.h"

#include "Sensor.h"

#include "PWM.h"

#include "LED.h"

#include "Buzzer.h"

#include "Encoder.h"

#include <stdint.h>

/* ===================== 第四问路线说明 ===================== */

/*

* 第四问路线：

*

* 初始：小车放在 A 点，车头朝 B 点

*

* 每一圈路线：

* A_ALIGN_TO_C：先把车头从 A->B 方向调整到 A->C 斜线方向

* A_TO_C：无黑线斜直线到 C，寻到黑线后声光提示

* C_ALIGN_TO_B：到 C 后先摆正到 C->B 半圆入口方向

* C_TO_B：沿半圆黑线循迹到 B，检测不到黑线后声光提示

* B_ALIGN_TO_D：B 点出弯后调整到 B->D 斜线方向

* B_TO_D：无黑线斜直线到 D，寻到黑线后声光提示

* D_ALIGN_TO_A：到 D 后先往 D->A 黑线方向修正一点

* D_TO_A：沿半圆黑线循迹到 A，检测不到黑线后声光提示

*

* 到 A 点后：

* 如果未完成 4 圈，则重新进入 A_ALIGN_TO_C，再次转向 C 点；

* 如果完成 4 圈，则停车。

*

* A 点出发角度：

* 第一次从起点 A 出发：-38 度

* 后面每次回到 A 后再出发：-45 度

*/

/* ===================== 基础速度参数 ===================== */

#define BASE_SPEED                  300

/*

* 半圆循迹速度降低一点，提高出弯稳定性。

*/

#define ARC_SPEED                   210

#define MAX_TIME_MS                 180000U

#define TOTAL_LAPS                  4

/* ===================== 编码器距离参数 ===================== */

#define COUNTS_PER_CM               20

/*

* A->C / B->D 仍然保留最小距离判断。

* 作用：避免刚出发时误识别黑线。

*

* 注意：

* AC_BACKUP_DISTANCE_CM 和 BD_BACKUP_DISTANCE_CM 现在不再用于强制结束，

* A->C / B->D 只会在寻到黑线后结束。

*/

#define DIAGONAL_TO_ARC_MIN_CM      75

#define AC_BACKUP_DISTANCE_CM       92

#define BD_BACKUP_DISTANCE_CM       93

/*

* 半圆出弯判断距离稍微加大，避免过早判断丢线。

*/

#define CB_ARC_MIN_DISTANCE_CM      95

#define DA_ARC_MIN_DISTANCE_CM      95

/* ===================== A 点 / C 点 / B 点 / D 点角度参数 ===================== */

/*

* 第一次从起点 A 出发用 -38 度。

* 后面每次回到 A 后再出发用 -45 度。

*/

#define A_TO_C_TARGET_DEG_FIRST     -38.0f

#define A_TO_C_TARGET_DEG_NEXT      -50.0f

#define C_TO_B_TARGET_DEG           60.0f

/*

* B 点出弯角度。

*/

#define B_OUT_BIAS_DEG              29.0f//30

/*

* B->D 到 D 后，先往 D->A 黑线方向修正一点。

*/

#define D_TO_A_TARGET_DEG           -51.0f

/* ===================== 直线修正参数 ===================== */

#define ENCODER_KP                  1

#define YAW_KP                      2

#define MAX_ENCODER_CORRECTION      60

#define MAX_YAW_CORRECTION          50

#define MAX_STRAIGHT_CORRECTION     100

/* ===================== 灰度循迹参数 ===================== */

/*

* 灰度循迹由原来的 P 控制改成 PD 控制：

* P 项负责跟线；

* D 项负责抑制左右摆动，让出弯角度更稳定。

*/

#define LINE_KP_ARC                 40

#define LINE_KD_ARC                 12

#define MAX_LINE_CORRECTION_ARC     110

#define LINE_ON_CONFIRM             2

#define LINE_LOST_CONFIRM           8U

#define CB_LINE_REVERSE             0

#define DA_LINE_REVERSE             0

/* ===================== 角度摆正基础参数 ===================== */

#define ALIGN_FORWARD_SPEED         130

#define ALIGN_KP                    4

#define ALIGN_OK_DEG                5.0f

#define ALIGN_REVERSE               1

/* ===================== A / C / B / D 点专用摆正参数 ===================== */

/*

* A 点专用：

* 如果出 A 角度变化不明显，优先加大 A_ALIGN_MAX_TIME_MS。

*/

#define A_ALIGN_MAX_TIME_MS         1500

#define A_ALIGN_MAX_CORRECTION      90

/*

* C 点专用：

* C 点主要用于进入 C->B 半圆。

*/

#define C_ALIGN_MAX_TIME_MS         1300

#define C_ALIGN_MAX_CORRECTION      80

/*

* B 点专用：

* 如果出 B 角度变化不明显，优先加大 B_ALIGN_MAX_TIME_MS。

*/

#define B_ALIGN_MAX_TIME_MS         1500

#define B_ALIGN_MAX_CORRECTION      90

/*

* D 点专用：

* D 点用于进入 D->A 半圆。

*/

#define D_ALIGN_MAX_TIME_MS         1500

#define D_ALIGN_MAX_CORRECTION      90

/* ===================== 全局变量 ===================== */

static volatile uint32_t g_ms = 0;

typedef enum

{

STATE_A_ALIGN_TO_C = 0,

STATE_A_TO_C,

STATE_C_ALIGN_TO_B,

STATE_C_TO_B,

STATE_B_ALIGN_TO_D,

STATE_B_TO_D,

STATE_D_ALIGN_TO_A,

STATE_D_TO_A,

STATE_STOP

} CarState_t;

static CarState_t g_state = STATE_A_ALIGN_TO_C;

static uint32_t g_start_time = 0;

static uint32_t g_state_time = 0;

static float g_yaw_start = 0.0f;

static float g_align_target = 0.0f;

static uint8_t g_line_on_count = 0;

static uint8_t g_line_lost_count = 0;

static int8_t g_last_line_error = 0;

/*

* 灰度循迹修正量滤波变量。

*/

static int32_t g_arc_correction_last = 0;

static uint8_t g_lap_count = 0;

/* ===================== Debug 监视变量 ===================== */

volatile uint8_t debug_state = 0;

volatile uint8_t debug_lap_count = 0;

volatile uint8_t debug_gray = 0;

volatile uint8_t debug_line_hit_count = 0;

volatile int8_t debug_line_error = 0;

volatile int8_t debug_line_d_error = 0;

volatile int16_t debug_left_speed = 0;

volatile int16_t debug_right_speed = 0;

volatile int32_t debug_left_encoder = 0;

volatile int32_t debug_right_encoder = 0;

volatile int32_t debug_avg_encoder = 0;

volatile float debug_yaw = 0.0f;

volatile float debug_relative_yaw = 0.0f;

volatile float debug_align_error = 0.0f;

/* ===================== SysTick ===================== */

void SysTick_Handler(void)

{

g_ms++;

}

static void Delay_ms(uint32_t ms)

{

uint32_t start = g_ms;

while ((g_ms - start) < ms)

{

JY901S_Update();

}

}

/* ===================== 工具函数 ===================== */

static int32_t Abs32(int32_t x)

{

return (x < 0) ? -x : x;

}

static float AbsFloat(float x)

{

return (x < 0.0f) ? -x : x;

}

static int16_t Limit_Int16(int32_t value, int16_t min, int16_t max)

{

if (value > max)

{

return max;

}

if (value < min)

{

return min;

}

return (int16_t)value;

}

static float Normalize_Angle(float angle)

{

while (angle > 180.0f)

{

angle -= 360.0f;

}

while (angle < -180.0f)

{

angle += 360.0f;

}

return angle;

}

static int32_t Get_Avg_Encoder(void)

{

int32_t left;

int32_t right;

left = Abs32(Encoder_GetLeft());

right = Abs32(Encoder_GetRight());

return (left + right) / 2;

}

static uint8_t Distance_OK(uint16_t cm)

{

int32_t target_count;

target_count = (int32_t)cm * COUNTS_PER_CM;

return (Get_Avg_Encoder() >= target_count);

}

static float Get_Relative_Yaw(void)

{

JY901S_Update();

if (!JY901S_HasYaw())

{

return 0.0f;

}

return Normalize_Angle(JY901S_GetYaw() - g_yaw_start);

}

/* ===================== 声光提示 ===================== */

static void Alert_Once(void)

{

Motor_Stop();

LED_On();

Buzzer_On();

Delay_ms(120);

LED_Off();

Buzzer_Off();

Delay_ms(80);

}

/* ===================== 状态切换 ===================== */

static void Begin_State(CarState_t next)

{

g_state = next;

g_state_time = g_ms;

Encoder_Clear();

g_line_on_count = 0;

g_line_lost_count = 0;

g_last_line_error = 0;

g_arc_correction_last = 0;

JY901S_Update();

if (JY901S_HasYaw())

{

g_yaw_start = JY901S_GetYaw();

}

debug_state = (uint8_t)g_state;

}

/* ===================== 七路灰度 ===================== */

static uint8_t Count_Gray_Hit(uint8_t gray)

{

uint8_t count = 0;

for (uint8_t i = 0; i < 7; i++)

{

if (gray & (1 << i))

{

count++;

}

}

return count;

}

static uint8_t Is_Line_Detected(void)

{

uint8_t gray;

gray = Gray_Read_All();

debug_gray = gray;

debug_line_hit_count = Count_Gray_Hit(gray);

return (debug_line_hit_count >= 1);

}

static uint8_t Line_On_Stable(void)

{

if (Is_Line_Detected())

{

if (g_line_on_count < LINE_ON_CONFIRM)

{

g_line_on_count++;

}

}

else

{

g_line_on_count = 0;

}

return (g_line_on_count >= LINE_ON_CONFIRM);

}

static uint8_t Line_Lost_Stable(void)

{

if (!Is_Line_Detected())

{

if (g_line_lost_count < LINE_LOST_CONFIRM)

{

g_line_lost_count++;

}

}

else

{

g_line_lost_count = 0;

}

return (g_line_lost_count >= LINE_LOST_CONFIRM);

}

/* ===================== 斜直线结束判断 ===================== */

/*

* A->C / B->D：

*

* 现在不再使用备用距离强制结束。

* 小车转完角度后一直往前走，

* 跑够 DIAGONAL_TO_ARC_MIN_CM 后，

* 只有稳定检测到黑线，才认为到达 C / D。

*

* 这样 C 点、D 点一定是寻到黑线后才声光提示。

*/

static uint8_t Diagonal_To_Arc_End_OK(uint16_t backup_cm)

{

(void)backup_cm;

if (Distance_OK(DIAGONAL_TO_ARC_MIN_CM) && Line_On_Stable())

{

return 1;

}

return 0;

}

/* ===================== 半圆结束判断 ===================== */

static uint8_t Arc_End_By_LineLost_OK(uint16_t min_cm)

{

if (Distance_OK(min_cm) && Line_Lost_Stable())

{

return 1;

}

return 0;

}

/* ===================== 无黑线直线控制 ===================== */

static void Straight_Run_No_Line(uint16_t base_speed)

{

int32_t left_count;

int32_t right_count;

int32_t encoder_error;

int32_t encoder_correction;

int32_t yaw_correction;

int32_t correction;

int16_t left_speed;

int16_t right_speed;

float yaw_error;

JY901S_Update();

left_count = Abs32(Encoder_GetLeft());

right_count = Abs32(Encoder_GetRight());

encoder_error = left_count - right_count;

encoder_correction = encoder_error * ENCODER_KP;

encoder_correction = Limit_Int16(

encoder_correction,

-MAX_ENCODER_CORRECTION,

MAX_ENCODER_CORRECTION

);

yaw_correction = 0;

if (JY901S_HasYaw())

{

yaw_error = Get_Relative_Yaw();

yaw_correction = (int32_t)(yaw_error * YAW_KP);

yaw_correction = Limit_Int16(

yaw_correction,

-MAX_YAW_CORRECTION,

MAX_YAW_CORRECTION

);

}

correction = encoder_correction + yaw_correction;

correction = Limit_Int16(

correction,

-MAX_STRAIGHT_CORRECTION,

MAX_STRAIGHT_CORRECTION

);

left_speed = base_speed - correction;

right_speed = base_speed + correction;

left_speed = Limit_Int16(left_speed, 0, 1000);

right_speed = Limit_Int16(right_speed, 0, 1000);

Motor_SetSpeed(left_speed, right_speed);

debug_left_speed = left_speed;

debug_right_speed = right_speed;

}

/* ===================== 灰度循迹 ===================== */

static int8_t Get_Line_Error(uint8_t gray)

{

int16_t sum = 0;

int8_t count = 0;

if (gray & (1 << 6)) { sum += -3; count++; }

if (gray & (1 << 5)) { sum += -2; count++; }

if (gray & (1 << 4)) { sum += -1; count++; }

if (gray & (1 << 3)) { sum +=  0; count++; }

if (gray & (1 << 2)) { sum +=  1; count++; }

if (gray & (1 << 1)) { sum +=  2; count++; }

if (gray & (1 << 0)) { sum +=  3; count++; }

if (count == 0)

{

return 0;

}

return (int8_t)(sum / count);

}

static void Line_Follow_Arc(uint16_t base_speed, uint8_t reverse)

{

uint8_t gray;

int8_t line_error;

int8_t d_error;

int32_t correction;

int16_t left_speed;

int16_t right_speed;

JY901S_Update();

gray = Gray_Read_All();

debug_gray = gray;

debug_line_hit_count = Count_Gray_Hit(gray);

line_error = Get_Line_Error(gray);

/*

* PD 控制：

* P 项：当前偏差越大，修正越大；

* D 项：偏差变化越快，反向抑制越强，用来减少左右摆动。

*/

d_error = line_error - g_last_line_error;

correction = (int32_t)line_error * LINE_KP_ARC +

(int32_t)d_error * LINE_KD_ARC;

g_last_line_error = line_error;

correction = Limit_Int16(

correction,

-MAX_LINE_CORRECTION_ARC,

MAX_LINE_CORRECTION_ARC

);

/*

* 修正量滤波：

* 让左右轮速度变化不要太突然，减少灰度循迹左右晃动。

*/

correction = (g_arc_correction_last * 2 + correction) / 3;

g_arc_correction_last = correction;

if (reverse)

{

left_speed = base_speed - correction;

right_speed = base_speed + correction;

}

else

{

left_speed = base_speed + correction;

right_speed = base_speed - correction;

}

left_speed = Limit_Int16(left_speed, 0, 1000);

right_speed = Limit_Int16(right_speed, 0, 1000);

Motor_SetSpeed(left_speed, right_speed);

debug_line_error = line_error;

debug_line_d_error = d_error;

debug_left_speed = left_speed;

debug_right_speed = right_speed;

}

/* ===================== B 点出弯目标角度设置 ===================== */

static void Set_B_Align_Target(void)

{

float yaw;

yaw = Get_Relative_Yaw();

if (yaw >= 0.0f)

{

g_align_target = Normalize_Angle(180.0f + B_OUT_BIAS_DEG);

}

else

{

g_align_target = Normalize_Angle(-180.0f - B_OUT_BIAS_DEG);

}

}

/* ===================== 低速角度摆正 ===================== */

static uint8_t Align_To_Target_Custom(uint32_t max_time_ms, int16_t max_correction)

{

float yaw;

float error;

int32_t correction;

int16_t left_speed;

int16_t right_speed;

JY901S_Update();

yaw = Get_Relative_Yaw();

error = Normalize_Angle(g_align_target - yaw);

debug_align_error = error;

if (AbsFloat(error) <= ALIGN_OK_DEG)

{

Motor_Stop();

return 1;

}

if ((g_ms - g_state_time) >= max_time_ms)

{

Motor_Stop();

return 1;

}

correction = (int32_t)(error * ALIGN_KP);

correction = Limit_Int16(

correction,

-max_correction,

max_correction

);

#if ALIGN_REVERSE

left_speed  = ALIGN_FORWARD_SPEED - correction;

right_speed = ALIGN_FORWARD_SPEED + correction;

#else

left_speed  = ALIGN_FORWARD_SPEED + correction;

right_speed = ALIGN_FORWARD_SPEED - correction;

#endif

left_speed = Limit_Int16(left_speed, 0, 1000);

right_speed = Limit_Int16(right_speed, 0, 1000);

Motor_SetSpeed(left_speed, right_speed);

debug_left_speed = left_speed;

debug_right_speed = right_speed;

return 0;

}

static uint8_t Align_To_Target_A(void)

{

return Align_To_Target_Custom(A_ALIGN_MAX_TIME_MS, A_ALIGN_MAX_CORRECTION);

}

static uint8_t Align_To_Target_C(void)

{

return Align_To_Target_Custom(C_ALIGN_MAX_TIME_MS, C_ALIGN_MAX_CORRECTION);

}

static uint8_t Align_To_Target_B(void)

{

return Align_To_Target_Custom(B_ALIGN_MAX_TIME_MS, B_ALIGN_MAX_CORRECTION);

}

static uint8_t Align_To_Target_D(void)

{

return Align_To_Target_Custom(D_ALIGN_MAX_TIME_MS, D_ALIGN_MAX_CORRECTION);

}

/* ===================== Debug 更新 ===================== */

static void Debug_Update(void)

{

JY901S_Update();

debug_state = (uint8_t)g_state;

debug_lap_count = g_lap_count;

debug_left_encoder = Encoder_GetLeft();

debug_right_encoder = Encoder_GetRight();

debug_avg_encoder = Get_Avg_Encoder();

if (JY901S_HasYaw())

{

debug_yaw = JY901S_GetYaw();

debug_relative_yaw = Get_Relative_Yaw();

}

}

/* ===================== 主函数 ===================== */

int main(void)

{

SYSCFG_DL_init();

SysTick_Config(CPUCLK_FREQ / 1000);

Gray_Init();

Motor_Init();

LED_Init();

Buzzer_Init();

Encoder_Init();

JY901S_Init();

Encoder_Clear();

for (uint16_t i = 0; i < 100; i++)

{

JY901S_Update();

Delay_ms(5);

}

if (JY901S_HasYaw())

{

g_yaw_start = JY901S_GetYaw();

}

else

{

g_yaw_start = 0.0f;

}

g_start_time = g_ms;

g_state_time = g_ms;

g_lap_count = 0;

debug_lap_count = 0;

g_state = STATE_A_ALIGN_TO_C;

debug_state = (uint8_t)g_state;

while (1)

{

JY901S_Update();

Debug_Update();

if ((g_state != STATE_STOP) &&

((g_ms - g_start_time) >= MAX_TIME_MS))

{

Motor_Stop();

Alert_Once();

g_state = STATE_STOP;

debug_state = (uint8_t)g_state;

}

switch (g_state)

{

case STATE_A_ALIGN_TO_C:

/*

* 第一次从起点 A 出发用 -38 度。

* 后面每次回到 A 后再出发用 -45 度。

*/

if (g_lap_count == 0)

{

g_align_target = A_TO_C_TARGET_DEG_FIRST;

}

else

{

g_align_target = A_TO_C_TARGET_DEG_NEXT;

}

if (Align_To_Target_A())

{

Begin_State(STATE_A_TO_C);

}

break;

case STATE_A_TO_C:

Straight_Run_No_Line(BASE_SPEED);

if (Diagonal_To_Arc_End_OK(AC_BACKUP_DISTANCE_CM))

{

/*

* A->C：

* 寻到 C 点黑线后才声光提示。

*/

Alert_Once();

Begin_State(STATE_C_ALIGN_TO_B);

}

break;

case STATE_C_ALIGN_TO_B:

g_align_target = C_TO_B_TARGET_DEG;

if (Align_To_Target_C())

{

Begin_State(STATE_C_TO_B);

}

break;

case STATE_C_TO_B:

Line_Follow_Arc(ARC_SPEED, CB_LINE_REVERSE);

if (Arc_End_By_LineLost_OK(CB_ARC_MIN_DISTANCE_CM))

{

/*

* C->B：

* 检测不到黑线后声光提示。

*/

Alert_Once();

Set_B_Align_Target();

g_state = STATE_B_ALIGN_TO_D;

g_state_time = g_ms;

debug_state = (uint8_t)g_state;

}

break;

case STATE_B_ALIGN_TO_D:

if (Align_To_Target_B())

{

Begin_State(STATE_B_TO_D);

}

break;

case STATE_B_TO_D:

Straight_Run_No_Line(BASE_SPEED);

if (Diagonal_To_Arc_End_OK(BD_BACKUP_DISTANCE_CM))

{

/*

* B->D：

* 寻到 D 点黑线后才声光提示。

*/

Alert_Once();

Begin_State(STATE_D_ALIGN_TO_A);

}

break;

case STATE_D_ALIGN_TO_A:

g_align_target = D_TO_A_TARGET_DEG;

if (Align_To_Target_D())

{

Begin_State(STATE_D_TO_A);

}

break;

case STATE_D_TO_A:

Line_Follow_Arc(ARC_SPEED, DA_LINE_REVERSE);

if (Arc_End_By_LineLost_OK(DA_ARC_MIN_DISTANCE_CM))

{

/*

* D->A：

* 检测不到黑线后声光提示，认为回到 A 点。

*/

Alert_Once();

g_lap_count++;

debug_lap_count = g_lap_count;

/*

* 如果完成 4 圈，停车。

* 如果未完成 4 圈，则重新进入 A_ALIGN_TO_C，

* 也就是到 A 后再次转向 C 点。

*/

if (g_lap_count >= TOTAL_LAPS)

{

g_state = STATE_STOP;

debug_state = (uint8_t)g_state;

}

else

{

Begin_State(STATE_A_ALIGN_TO_C);

}

}

break;

case STATE_STOP:

default:

Motor_Stop();

break;

}

Delay_ms(10);

}

}